American Football Launcher Design Baseline

Sections 1-4: Stakeholders, CONOPS, Product Design Specification v0.1, Requirements Traceability Matrix and RFLP Decomposition

# Phase 1 — Stakeholders, Mission Need and Use Case

This phase defines the reason for the project, the people affected by the design and the high-level use case that should guide concept development.

## 1.1 Stakeholders

| Stakeholder | What they care about |
| --- | --- |
| Operator | Safety, ease of loading and simple controls. |
| Student designer | Demonstrable engineering rigour, clear design decisions and test evidence. |
| Assessor / future employer | Evidence of CAD, analysis, trade-offs, DFMA, validation and documentation. |
| Nearby users / bystanders | Guarding, controlled launch direction and safe operating procedure. |
| Future maintainer | Simple assembly, replaceable parts and clear documentation. |

## 1.2 Mission Need

Manual American football throwing is inconsistent and skill-dependent. The project aims to develop a compact mechanical launcher capable of producing repeatable ball speed, launch angle and spiral spin while remaining safe, manufacturable and suitable for student-level prototyping.

## 1.3 Use Case Boundary

The prototype is intended for controlled testing and engineering demonstration, not unsupervised public use. The early design should prioritise repeatability, safe operation, standard components, simple manufacture and measurable validation over maximum launch power.

# Phase 2 — Concept of Operations

The concept of operations defines how the American football launcher is expected to be used during normal operation and how the system should respond to unsafe or fault conditions. This is included before detailed CAD development so that the design is driven by function, safety and verification rather than by arbitrary geometry.

## 2.1 CONOPS Narrative

The launcher is intended to be placed on stable ground, adjusted for launch angle and direction, loaded with the football in a repeatable orientation, guarded, spun up to the target operating condition, armed, checked for safety and then released. After launch, the carriage is arrested, the spin module powers down and the system is reset for the next cycle.

## 2.2 Normal Operating Sequence

Place launcher on stable ground.

Set launch angle and direction.

Open guard or loading access.

Insert football laces-up into cradle/indexing feature.

Set ball orientation so it is nose-forward and repeatably indexed.

Adjust contactor preload onto ball.

Close guard.

Spin module accelerates to target RPM.

Elastic carriage is armed.

System confirms safe state: guard closed, operator clear and launch path clear.

Release carriage.

Ball accelerates forward through the spin/contact module.

Ball leaves the launcher before the carriage hits the end stop.

Carriage is arrested by a bumper/brake.

Spin module powers down.

System resets.

## 2.3 Fault and Safety Sequence

| Fault | Required response |
| --- | --- |
| Guard opened | Motor off; release blocked. |
| Emergency stop pressed | Motor off; release blocked. |
| Ball misloaded | No launch; operator re-loads and re-checks orientation. |
| Overspeed detected | Motor disabled; inspect spin module before further testing. |
| Carriage not reset | Launch inhibited until carriage is correctly reset and armed. |
| Excessive vibration | Stop test; inspect rotating assembly, contactors and fixtures. |

## 2.4 Normal and Fault Operating Flow

Figure 1. Concept of operations flowchart for the football launcher.

This flowchart shows the full operating sequence from setup to reset. Key safety checks are included before spin-up, arming and launch. Unsafe conditions such as an open guard, overspeed, incorrect carriage reset or unsafe launch path move the system into an inhibited or fault state rather than allowing release.

## 2.5 System State Diagram

Figure 2. State diagram showing normal, inhibited and fault states.

This diagram represents the launcher as a sequence of operating states rather than individual user actions. It highlights that the system should only enter the launch state after loading, guarding, spin-up, arming and final safety checks have been completed. Any safety-critical fault should move the system into a fault or safe-inhibit state, where motor power is removed and release is blocked until the fault has been cleared.

## 2.6 Design Implications from the CONOPS

The CONOPS creates several direct design requirements. The launcher requires a repeatable ball indexing feature, an adjustable contactor preload mechanism, a guard around rotating components, a controlled spin-up process, a carriage arming and release system, and a carriage arrest system. It also implies the need for safety interlocks or prototype-equivalent manual checks for guard closure, carriage reset, overspeed, ball loading and launch path clearance.

For the early prototype, some checks may be implemented manually, but they should still be treated as formal safety conditions during testing. The final design should make the transition from Idle to Setup, Loading, Guarded, Spin-up, Armed, Ready to Launch, Launching and Recovery clear to the operator and difficult to bypass accidentally.

## 2.7 CONOPS V-Model and Verification Approach

The CONOPS V-model links the mission need and stakeholder expectations to the derived requirements, architecture and detailed design decisions that follow. It also shows the matching verification ladder, from inspection and component checks up to subsystem tests, full system verification, acceptance checks and operational validation.

Figure 3. CONOPS V-model linking operational intent, derived design definition and planned verification evidence.

For this launcher, the practical verification strategy is to prove low-level items first by inspection, calculations and component tests, then progressively integrate the frame, drive, controls and release mechanism before running full launch demonstrations. That makes this subsection the bridge between the Phase 2 CONOPS and the measurable PDS requirements in Phase 3, and it sets up the requirement-by-requirement traceability matrix added in Phase 4.

# Phase 3 — Product Design Specification v0.1

The Product Design Specification translates the stakeholder needs and CONOPS into measurable requirements. The numerical targets are initial design targets and should be revisited after first-principles calculations, component selection and early prototype tests.

| ID | Requirement | Target | Priority | Rationale | Verification / pass-fail rule |
| --- | --- | --- | --- | --- | --- |
| REQ-01 | The launcher shall produce a measurable football exit speed. | 10–20 m/s | Must | Main performance requirement; defines whether the launcher actually performs its function. | Measure 5 launches using high-speed video or timed distance. Pass if at least 4/5 launches are within target range. |
| REQ-02 | The launcher shall achieve a useful throw range. | 10–30 yards | Must | Demonstrates practical throwing capability without making the prototype dangerously powerful. | Field test with marked distance. Pass if ball lands within target range in controlled tests. |
| REQ-03 | The launcher shall generate spiral spin. | 600–1000 rpm initial target | Must | Spiral stability is central to an American football launcher, not just projectile launching. | Mark ball and use slow-motion video. Pass if rotation can be measured and remains broadly axial. |
| REQ-04 | The ball shall exit nose-forward. | Nose-forward at release | Must | Poor orientation would make the spin module irrelevant and reduce repeatability. | Side-view video inspection. Pass if nose-forward release occurs in at least 4/5 tests. |
| REQ-05 | The ball contact system shall use a symmetric contact layout. | 3 contactors approx. 120° apart | Should | Symmetric contact reduces lateral force imbalance and supports controlled spin. | CAD inspection and physical measurement. Pass if angular spacing is within acceptable tolerance, e.g. ±10°. |
| REQ-06 | The contactor preload shall be adjustable. | Approx. 10–25 N per contactor | Must | Allows tuning for different football stiffness, grip and deformation. | Spring scale/load-cell test. Pass if preload can be adjusted and measured across target range. |
| REQ-07 | The ball shall leave the launcher before carriage arrest. | Clear exit before bumper/end-stop contact | Must | Prevents the carriage/end stop from disturbing launch or damaging the ball/system. | Slow-motion side-view video. Pass if ball exit occurs before carriage arrest in all safe tests. |
| REQ-08 | The launch angle shall be adjustable. | 10–45° | Should | Allows testing of range and trajectory without redesigning the frame. | Measure with angle gauge/protractor. Pass if angle can be set and held at selected positions. |
| REQ-09 | The design shall prevent user contact with rotating parts during operation. | No finger access to spin module when powered | Must | Essential safety requirement. | Physical inspection. Pass if guarded access prevents contact during powered operation. |
| REQ-10 | The system shall include an emergency stop or equivalent shutdown control. | Motor off and release blocked | Must | Required because the system contains rotating parts and stored elastic energy. | Functional test. Pass if emergency stop disables motor and prevents release. |
| REQ-11 | The design shall prioritise standard bought-in components. | Bearings, rails, fasteners and motors bought where possible | Should | Reduces manufacturing difficulty and shows good integration of standard parts. | BOM review. Pass if major motion/support elements are standard parts unless justified. |
| REQ-12 | The design shall be manufacturable with student-accessible methods. | 3D printed parts, simple plates, bought-in rails/shafts | Must | Keeps the prototype realistic for time, budget and workshop access. | Drawing/process review. Pass if each custom part has a defined manufacturing method. |
| REQ-13 | The launcher shall produce repeatable exit speed. | ±10–15% variation over 5 launches | Must | Repeatability is the real purpose of replacing manual throwing. | Measure 5 launches at same settings. Pass if speed variation remains within chosen tolerance. |
| REQ-14 | The launcher shall produce repeatable launch direction. | Defined target zone at fixed range | Should | Shows the system is controlled rather than just powerful. | Launch 5 balls at fixed settings. Pass if majority land within a defined test zone. |
| REQ-15 | The system shall inhibit launch when the guard is open. | No motor/release action with guard open | Must | Converts the safety concept into a testable requirement. | Functional interlock test. Pass if launch cannot occur while guard is open. |
| REQ-16 | The system shall limit spin-module speed. | Maximum RPM to be defined after motor selection | Must | Prevents overspeed, vibration and rotating-part failure. | Tachometer/ESC setting check. Pass if speed limit is implemented and tested. |
| REQ-17 | The system shall have a defined maximum stored energy. | To be calculated from elastic/carriage design | Must | Stored elastic energy is a major hazard and drives frame/stop design. | Calculation and inspection. Pass if stored energy is calculated and safety factor/design controls are documented. |
| REQ-18 | The carriage arrest system shall absorb launch energy without structural damage. | No cracking, detachment or unsafe rebound | Must | End-stop failure is one of the most likely prototype hazards. | Low-energy tests progressing to full-energy tests. Pass if no visible damage or uncontrolled rebound occurs. |
| REQ-19 | The prototype shall remain stable during launch. | No tipping, sliding or uncontrolled movement | Must | A launcher that moves during firing is unsafe and invalidates accuracy tests. | Video/physical test. Pass if frame remains stable during repeated launches. |
| REQ-20 | The launcher shall be transportable by one person or two people. | Mass/envelope to be defined after CAD layout | Could | Useful practical constraint but secondary to safety/performance. | Weighing and handling trial. Pass if transport method is practical and safe. |
| REQ-21 | The prototype shall have a defined power supply and electrical limit. | Voltage/current limit matched to motor and controller | Must | Prevents overloading electronics and makes tests repeatable. | Electrical review. Pass if supply, ESC/controller and motor ratings are documented. |
| REQ-22 | The design shall include a basic test plan. | Speed, range, spin, preload and safety checks | Must | Converts design claims into evidence. | Test-plan review. Pass if each must-have requirement has a verification method. |
| REQ-23 | The design shall include a preliminary risk assessment. | Hazards, likelihood, severity and mitigation | Must | Necessary due to rotating parts, stored energy and projectile launch. | Risk-table review. Pass if all major hazards are identified with mitigations. |
| REQ-24 | The final design package shall include CAD, BOM and drawings. | Assembly CAD, part drawings and BOM | Should | Shows engineering process and makes the project useful as CV evidence. | Documentation review. Pass if another person could understand and rebuild the concept. |

## 3.1 Requirement Grouping

The same requirements are grouped below to make the design work packages easier to plan and verify.

### Performance requirements

| ID | Requirement |
| --- | --- |
| REQ-01 | Launch speed: 10–20 m/s |
| REQ-02 | Range: 10–30 yards |
| REQ-03 | Spiral spin: 600–1000 rpm |
| REQ-04 | Nose-forward release |
| REQ-13 | Exit speed repeatability |
| REQ-14 | Direction/range repeatability |

### Mechanical design requirements

| ID | Requirement |
| --- | --- |
| REQ-05 | 3 contactors approximately 120° apart |
| REQ-06 | Adjustable preload |
| REQ-07 | Ball exits before carriage arrest |
| REQ-08 | Adjustable launch angle |
| REQ-17 | Maximum stored energy calculated |
| REQ-18 | Carriage arrest survives launch loads |
| REQ-19 | Frame remains stable during launch |

### Safety requirements

| ID | Requirement |
| --- | --- |
| REQ-09 | Rotating parts guarded |
| REQ-10 | Emergency stop/shutdown |
| REQ-15 | Guard-open launch inhibit |
| REQ-16 | Spin-module overspeed limit |
| REQ-23 | Risk assessment completed |

### Manufacture and integration requirements

| ID | Requirement |
| --- | --- |
| REQ-11 | Use standard bought-in components where possible |
| REQ-12 | Custom parts manufacturable by 3D printing/simple machining |
| REQ-20 | Prototype transportability |
| REQ-21 | Defined power supply and electrical limits |
| REQ-24 | CAD, BOM and drawings completed |

### Test and validation requirements

| ID | Requirement |
| --- | --- |
| REQ-22 | Test plan covering speed, range, spin, preload and safety |
| REQ-01 to REQ-24 | Each requirement has a verification method |

# Phase 4 — Requirements Traceability Matrix and RFLP Decomposition

This phase translates the V-model into working engineering traceability. Each Phase 3 requirement is linked back to its CONOPS origin and forward to the intended design response, verification method and evidence that should exist once the prototype has been analysed or tested.

## 4.1 Requirement Traceability Matrix

The matrix below provides the bridge from operational intent to design action. It should be used to check that every requirement can be justified, implemented, verified and supported by a concrete evidence item.

| PDS ID | CONOPS origin | Design response | Verification method | Evidence |
| --- | --- | --- | --- | --- |
| REQ-01 | 1.2 Mission need for repeatable launch speed; 2.1/2.2 controlled spin-up and release | Motor, controller and contact geometry sized to deliver target exit speed | Five-shot speed test by high-speed video or timed distance | Shot log, calculation sheet and test video |
| REQ-02 | 1.2 Useful throwing capability; 2.2 adjustable angle and controlled release | Adjustable launch angle, tuned energy input and stable frame | Field range test at marked distances | Range test sheet and landing map/video |
| REQ-03 | 1.2 Spiral spin need; 2.2 indexed loading and spin-up | Symmetric spin/contact module with controlled RPM and indexing | Slow-motion video with marked ball to measure RPM and spin axis | Annotated video and RPM estimate sheet |
| REQ-04 | 2.2 Repeatable indexed orientation before release | Cradle/indexing feature that sets nose-forward orientation | Side-view release video over five launches | Release video and observation log |
| REQ-05 | 2.6 Need for repeatable ball indexing/contact | Three-contact symmetric geometry around the ball path | CAD inspection and physical angular measurement | CAD screenshot and measurement record |
| REQ-06 | 2.2 Preload adjustment step; 2.6 adjustable preload implication | Adjustable spring or screw preload mechanism at contactors | Spring-scale or load-cell preload check across range | Preload calibration sheet and photos |
| REQ-07 | 2.2 Ball exits before carriage arrest; 2.6 need for carriage arrest | Release timing and travel set so the ball clears before bumper engagement | Slow-motion side-view video during launch | Timed video frames and test note |
| REQ-08 | 2.1/2.2 Setup requires launch-angle adjustment | Angle-setting mechanism with lockable positions and scale | Protractor or angle-gauge check at selected positions | Angle-setting record and photos |
| REQ-09 | 1.3 Controlled prototype use; 2.3/2.4 guard fault response; 2.6 guard implication | Fixed or hinged guard enclosing rotating parts and nip points | Physical inspection during powered-ready condition | Guard inspection checklist and photos |
| REQ-10 | 2.3 Safety sequence; 2.4 fault flow to motor-off and release-blocked state | Emergency stop or equivalent shutdown path removing motor power and blocking release | Functional emergency-stop test | Functional test record and demo video |
| REQ-11 | 1.3 Student prototype and simple manufacture intent | Select standard motors, bearings, rails, fasteners and off-the-shelf hardware | BOM and design review | BOM extract and sourcing notes |
| REQ-12 | 1.3 Student-level prototyping; 2.6 prototype-equivalent controls | Use 3D-printed or simple machined custom parts with standard interfaces | Drawing and process review | Manufacturing notes, drawings and part list |
| REQ-13 | 1.2 Repeatability objective; 2.1 reset and controlled sequence | Consistent preload, indexing and drive-speed control at fixed settings | Five-shot exit-speed repeatability test | Speed data table and variation calculation |
| REQ-14 | 1.2 Repeatable direction; 2.1 stable setup and alignment | Rigid frame, guided launch path and repeatable aim setup | Five-shot landing-dispersion test | Target map, shot plot and video |
| REQ-15 | 2.3 Guard-open fault; 2.4 inhibited-state logic | Guard interlock or procedural lockout that blocks motor and release | Open-guard interlock functional test | Interlock test sheet and photo/video |
| REQ-16 | 2.3 Overspeed fault response; 2.6 need for overspeed check | Controller RPM limit, tachometer feedback or fixed speed cap | Tachometer check and controller settings review | RPM log and controller configuration record |
| REQ-17 | 2.3 Unsafe stored-energy concern; 2.6 carriage and elastic-system implication | Defined spring or elastic energy budget, safety factor and guarded energy path | Stored-energy calculation and design inspection | Calculation sheet and design-review sign-off |
| REQ-18 | 2.2 Carriage arrest step; 2.6 carriage arrest-system implication | Bumper and end-stop structure sized to absorb carriage energy safely | Progressive low-to-full-energy arrest tests | Test photos/video and post-test inspection record |
| REQ-19 | 2.1 Stable-ground placement; 2.2 first operating step | Wide base, anchors or feet and mass distribution that resist tipping/sliding | Repeated launch stability test | Launch video and stability checklist |
| REQ-20 | 1.1 Operator and student stakeholder practicality; 1.3 compact prototype intent | Manage frame size, module packaging and handling points | Weighing and handling trial | Mass record and handling assessment |
| REQ-21 | 2.1 Controlled spin-up; 2.3 electrical shutdown safety | Defined supply voltage/current, protected controller and rated wiring | Electrical design review against motor/controller ratings | Electrical schematic, ratings sheet and review note |
| REQ-22 | 2.7 Verification approach; 3.1 test and validation grouping | Test plan covering speed, range, spin, preload and safety checks | Test-plan completeness review | Controlled test plan and requirements checklist |
| REQ-23 | 2.3 Fault and safety sequence; 2.4 fault states | Preliminary hazard log with mitigations, interlocks and safe procedures | Risk-assessment review | Risk table with mitigations and owner/date |
| REQ-24 | 2.6 design implications; Phase 3 PDS baseline | Produce assembly CAD, part drawings, BOM and interface details | Design-package documentation review | CAD files, drawing pack and BOM |

## 4.2 RFLP Systems Decomposition

The RFLP decomposition makes the systems architecture explicit by separating what the launcher must achieve from how the solution is eventually built. The requirement view is already established by the PDS and traceability matrix. The functional, logical and physical views below convert the CONOPS into a structure that can be used for concept selection, CAD layout and subsystem analysis.

For this project, the RFLP structure is used as a practical design-control tool rather than as a software exercise. Each physical feature should be traceable back to a logical subsystem, a required function and one or more PDS requirements.

### 4.2.1 Requirement View

| Requirement group | Linked PDS IDs | Meaning for the design |
| --- | --- | --- |
| Performance | REQ-01, REQ-02, REQ-03, REQ-04, REQ-13, REQ-14 | The launcher must generate useful forward speed, range, spiral spin, nose-forward release and repeatable output. |
| Mechanical architecture | REQ-05, REQ-06, REQ-07, REQ-08, REQ-17, REQ-18, REQ-19 | The mechanism must provide controlled contact geometry, adjustable preload, clean release timing, stable support and safe carriage arrest. |
| Safety and controls | REQ-09, REQ-10, REQ-15, REQ-16, REQ-21, REQ-23 | The design must protect the operator from rotating parts, stored energy, overspeed and unsafe launch states. |
| Manufacture and integration | REQ-11, REQ-12, REQ-20, REQ-24 | The system should use standard components where possible and remain feasible for student-accessible manufacture and documentation. |
| Verification and validation | REQ-22 and all PDS requirements | Each requirement must have a defined inspection, calculation, bench test or launch test that can produce evidence. |

### 4.2.2 Functional View

The functional view describes what the launcher must do without committing to a final physical solution.

| Function | Description |
| --- | --- |
| Support launcher | Provide a stable base and maintain the selected launch angle during operation. |
| Hold ball | Locate the football before launch so that loading is repeatable. |
| Index ball | Set a consistent nose-forward and laces-up orientation before launch. |
| Store launch energy | Store the energy needed to accelerate the ball forward. |
| Impart forward velocity | Accelerate the ball along the launch path to achieve the target exit speed and range. |
| Impart spiral spin | Apply controlled tangential contact to create axial ball rotation. |
| Release ball cleanly | Allow the ball to leave the contact/launch region without tumbling or being disturbed by the carriage. |
| Arrest carriage | Stop remaining carriage motion safely after the ball has exited. |
| Guard user | Prevent contact with rotating parts, nip points and stored-energy hazards. |
| Control operation | Manage spin-up, arming, release inhibit, shutdown and emergency-stop behaviour. |
| Reset system | Return the launcher to a safe repeatable state after each launch. |
| Measure performance | Capture speed, spin, range, repeatability and safety evidence during testing. |

### 4.2.3 Logical View

The logical view groups the functions into subsystems that can be designed, analysed and tested separately before integration.

| Logical subsystem | Main functions served | Key interfaces |
| --- | --- | --- |
| Base/frame | Support launcher, set launch angle, resist launch loads | Ground, rail carriage, guard, launch-angle adjustment |
| Ball cradle/indexer | Hold ball and index ball orientation | Football, carriage, loading access, operator |
| Rail carriage | Impart forward velocity and guide ball path | Rails, elastic drive, release latch, cradle, end stop |
| Elastic drive / launch energy system | Store and release launch energy | Frame, carriage, release latch, guarding |
| Release latch | Hold the armed carriage and release it when safe | Operator/control input, carriage, safety inhibit |
| Carriage arrest system | Absorb remaining carriage energy after ball exit | Carriage, frame, bumper/damper, guard |
| Spin module | Impart spiral spin through controlled contact | Football, contactors, motor/drive, preload adjusters |
| Motor/drive system | Spin contactors at controlled speed | Power supply, controller, belts/gears/shafts, spin module |
| Control and safety system | Control operation, inhibit unsafe states and perform shutdown | E-stop, guard switch, motor controller, release inhibit, operator |
| Guarding | Guard user from rotating and stored-energy hazards | Frame, spin module, loading access, interlock/safety procedure |
| Measurement system | Measure speed, spin, range and repeatability | Ball, launch path, camera/tachometer, test area |

### 4.2.4 Physical View

The physical view identifies practical implementation options for the logical subsystems. These are not yet final CAD decisions; they form the starting point for concept selection and architecture freeze.

| Logical subsystem | Candidate physical implementation | Notes for concept selection |
| --- | --- | --- |
| Base/frame | Aluminium extrusion, plywood prototype frame, steel brackets or reinforced 3D-printed structure | Must resist tipping, sliding and launch reaction loads. |
| Ball cradle/indexer | 3D-printed cradle or plate-based support with laces-up indexing feature | Must avoid damaging or over-constraining the football. |
| Rail carriage | Linear guide rail, drawer slide, aluminium extrusion rail or shaft-guided sled | Must move smoothly and release the ball before end-stop contact. |
| Elastic drive / launch energy system | Latex tubing, bungee cord, extension springs or spring-carriage system | Stored energy must be calculated and guarded. |
| Release latch | Mechanical pin latch, lever trigger, solenoid-assisted latch or manually released catch | Must not release accidentally and should be compatible with safety inhibit. |
| Carriage arrest system | Rubber bumper, foam damper, spring stop, friction brake or combined bumper/brake | Must absorb carriage energy without cracking or unsafe rebound. |
| Spin module | Three rubber contactors approximately 120 degrees apart, belt-driven rollers or rotating ring | Must produce spiral spin while allowing clean ball exit. |
| Motor/drive system | DC or brushless motor with ESC, belt reduction, pulley train or gear reduction | RPM and torque must be matched to spin target and speed limit. |
| Control and safety system | Arduino, Pico or ESP32 with motor ESC, emergency stop, guard switch and speed limit | Prototype can use manual checks initially, but safety states must remain formal. |
| Guarding | Polycarbonate, plywood or printed enclosure around rotating/contact regions | Must prevent finger access during powered operation. |
| Measurement system | Phone slow-motion video, marked ball, distance markers, tachometer or optical sensor | Must generate evidence for PDS verification. |

### 4.2.5 Interface and Architecture Diagram

Figure 4. RFLP logical interface diagram for the football launcher.

The interface diagram shows the main energy, control and safety paths before detailed CAD begins. The football is affected by two controlled physical actions: forward acceleration from the rail carriage and spiral generation from the spin module. The control and safety system should govern spin-up, release inhibit and emergency shutdown, while the base/frame and guarding provide structural support and hazard separation.

### 4.2.6 RFLP Exit Gate

The RFLP decomposition is complete enough to support concept generation once each major function has at least one logical subsystem and at least one plausible physical implementation. The remaining work is not to expand RFLP further, but to use it to compare concepts, define interfaces and prevent CAD features from being added without a requirement or function behind them.

This traceability matrix and RFLP decomposition now provide the hand-off into Phase 5 concept generation and selection, because alternative launcher concepts can be judged against traced requirements, required functions, subsystem interfaces and plausible physical implementations rather than against isolated features.

Phase 5 — Concept Generation and Selection

This phase compares alternative launcher concepts against the requirements, CONOPS and RFLP decomposition already developed in Phases 1–4. The purpose is to avoid selecting a design purely because it is visually attractive or easy to model in CAD. Each concept is judged against the functions the launcher must perform, the safety conditions it must satisfy, and the evidence that will later be needed for verification.

The baseline concept is treated as one option within the trade study, not as an automatic final answer. The selected concept will only be carried forward if it scores well against the PDS and if the next phase confirms that the energy, force, torque, contact and safety requirements are physically achievable.

5.1 Concepts Considered

| Concept | Name | Description |
| --- | --- | --- |
| A | Elastic rail carriage + three-contactor spin module | Stored elastic energy accelerates the ball forward on a guided carriage. A separate driven three-contactor spin module applies spiral spin before the ball exits. The ball should leave before the carriage reaches the arrest system. |
| B | Dual-wheel friction launcher | Two powered wheels grip and accelerate the football directly. Speed is controlled by motor speed and wheel contact force. Spiral may be attempted through angled wheel contact or differential wheel speed. |
| C | Pneumatic piston launcher + passive spin guide | A pneumatic piston or air-powered carriage provides forward launch energy. Passive angled guide surfaces or fins attempt to impart spiral spin during launch. |
| D | Catapult arm launcher with spin guide | A rotating arm throws the football forward. A guide, cradle or shaped release path attempts to control ball orientation and spiral spin. |
| E | Belt/roller acceleration tunnel | Belts or rollers accelerate the football through a guided channel. Angled or offset roller contact is used to generate spiral spin while the ball travels through the tunnel. |

5.2 Evaluation Criteria

The concepts are scored from 1 to 5 against each criterion.

| Score | Meaning |
| --- | --- |
| 1 | Very poor fit; major redesign or unacceptable risk likely. |
| 2 | Weak fit; possible but with significant limitations. |
| 3 | Acceptable but with clear concerns. |
| 4 | Strong fit with manageable risks. |
| 5 | Excellent fit; directly supports the requirement with low uncertainty. |

The weighting reflects the importance of each criterion to the project. High-priority criteria are linked to must-have requirements such as controlled launch speed, spiral generation, nose-forward release, safety, repeatability and test evidence.

| Criterion | Weight | Reason for weighting |
| --- | --- | --- |
| Launch speed/range capability | 5 | The launcher must produce useful forward velocity and range. |
| Spiral quality | 5 | Spiral spin is central to the project aim and distinguishes the design from a simple projectile launcher. |
| Nose-forward release | 5 | The ball must leave in a stable orientation for spin and repeatability to be meaningful. |
| Repeatability | 5 | Repeatability is the main reason for replacing manual throwing. |
| Safety | 5 | The system contains rotating parts, stored energy and projectile hazards. |
| Build feasibility | 5 | The prototype must be achievable with student-level time, tools and budget. |
| Testability | 5 | The design must allow speed, spin, range, preload and safety to be verified. |
| Manufacturability / standard parts | 4 | The design should use bought-in components where possible and avoid unnecessary custom complexity. |
| CAD / mechanical design evidence | 4 | The project should demonstrate real mechanical design, interfaces, calculations and drawings. |
| Cost | 4 | The prototype must remain realistic for a student project. |
| Control-system opportunity | 3 | Useful for engineering depth, but secondary to the mechanical launch function. |
| Compactness / transportability | 3 | Important, but less critical than safety, repeatability and performance. |
| Novelty / portfolio value | 4 | The design should show more originality than a generic launcher while remaining achievable. |

5.3 Detailed Scoring Matrix

| Criterion | Weight | A: Elastic rail + 3-contactor | B: Dual-wheel friction | C: Pneumatic + passive spin | D: Catapult arm + spin guide | E: Belt/roller tunnel |
| --- | --- | --- | --- | --- | --- | --- |
| Launch speed/range capability | 5 | 4 | 5 | 4 | 3 | 4 |
| Spiral quality | 5 | 5 | 3 | 2 | 2 | 4 |
| Nose-forward release | 5 | 5 | 3 | 2 | 2 | 4 |
| Repeatability | 5 | 4 | 4 | 3 | 2 | 4 |
| Safety | 5 | 3 | 3 | 2 | 2 | 3 |
| Build feasibility | 5 | 3 | 4 | 2 | 3 | 2 |
| Testability | 5 | 4 | 5 | 3 | 3 | 4 |
| Manufacturability / standard parts | 4 | 3 | 4 | 3 | 3 | 2 |
| CAD / mechanical design evidence | 4 | 5 | 3 | 3 | 4 | 5 |
| Cost | 4 | 3 | 4 | 2 | 4 | 2 |
| Control-system opportunity | 3 | 4 | 4 | 2 | 1 | 4 |
| Compactness / transportability | 3 | 3 | 4 | 3 | 3 | 2 |
| Novelty / portfolio value | 4 | 5 | 2 | 3 | 3 | 4 |

5.4 Weighted Results

| Rank | Concept | Weighted score | Percentage | Interpretation |
| --- | --- | --- | --- | --- |
| 1 | A: Elastic rail carriage + three-contactor spin module | 225 / 285 | 78.9% | Best overall fit. Strongest for spiral quality, nose-forward release, CAD evidence and novelty, but has complexity and safety risks. |
| 2 | B: Dual-wheel friction launcher | 211 / 285 | 74.0% | Very buildable and testable, but weaker for novelty and controlled football-specific spiral generation. |
| 3 | E: Belt/roller acceleration tunnel | 195 / 285 | 68.4% | Good theoretical control of speed and spin, but high complexity, cost and integration burden. |
| 4 | D: Catapult arm with spin guide | 153 / 285 | 53.7% | Cheap and visually simple, but weak for repeatability, spin control and safety. |
| 5 | C: Pneumatic piston + passive spin guide | 149 / 285 | 52.3% | Capable of forward launch, but poor fit for controlled spiral, repeatability, safety and student-level build risk. |

5.5 Concept-by-Concept Assessment

Concept A — Elastic rail carriage + three-contactor spin module

Concept A separates the forward launch function from the spin generation function. This is its main technical advantage. The elastic rail carriage provides forward velocity, while the three-contactor spin module can be tuned separately for preload, contact geometry and rotational speed. This directly supports the requirements for launch speed, spiral spin, nose-forward release, adjustable preload and repeatable testing.

Its main weakness is complexity. The concept requires a rail carriage, elastic energy storage, a release latch, carriage arrest, motor drive, rotating contactors, guarding and control/safety logic. It is therefore not the easiest concept to build. However, this complexity is also what gives the concept strong engineering value, provided it is managed through modular design, calculations, risk assessment and progressive testing.

Key risks:

- Excessive stored energy in the elastic launch system.

- Carriage arrest failure or unsafe rebound.

- Ball slip or jamming in the spin module.

- Poor alignment between carriage path and spin module.

- Increased CAD and assembly complexity.

Mitigations:

- Calculate stored energy before full-energy tests.

- Use progressive low-energy testing.

- Design the carriage arrest system as a separate verified subsystem.

- Make contactor preload adjustable.

- Use guarding and release inhibit logic.

- Build the CAD around a clear launch-axis datum.

Concept B — Dual-wheel friction launcher

Concept B is the most buildable alternative. It uses powered wheels to grip and accelerate the ball, making it easier to test speed, motor control and repeatability. It also benefits from available examples in pitching-machine-style mechanisms and can use standard motors, wheels, bearings and guards.

Its weakness is that an American football is not spherical. A dual-wheel system may launch the ball forward, but it may struggle to control nose-forward orientation and spiral spin unless the wheel geometry is carefully adapted. The concept risks becoming a generic ball launcher rather than a football-specific engineering solution.

Key risks:

- Ball deformation or inconsistent grip.

- Poor spiral control.

- Orientation sensitivity during loading.

- Wheel slip and wear.

- Less novelty than the baseline concept.

Mitigations:

- Add a football-shaped cradle or indexing ramp.

- Use adjustable wheel spacing and compliant wheel surfaces.

- Investigate angled or differential wheel speed for spin.

- Use this concept as a buildability benchmark against Concept A.

Concept C — Pneumatic piston launcher + passive spin guide

Concept C can generate forward launch energy effectively, but it scores poorly because spin generation is mostly passive. Pneumatic systems also introduce additional safety and integration issues, including pressure containment, valves, sealing, noise and control uncertainty. For a student-level prototype, the risk-to-benefit ratio is poor unless pneumatic hardware is already available and well understood.

Key risks:

- Pressure safety and leakage.

- Poor control of spiral spin.

- Extra hardware cost and complexity.

- Higher safety burden.

- Less direct mechanical design evidence for the spin system.

Mitigations:

- Use only low-pressure pneumatic components.

- Add pressure relief and guarding.

- Use passive spin only as a secondary comparison, not the preferred route.

Concept D — Catapult arm launcher with spin guide

Concept D is mechanically simple and potentially cheap, but it is weak against the most important football-specific requirements. A catapult arm can launch the ball, but controlling nose-forward release and spiral spin is difficult. The motion is also highly dynamic, which creates safety and repeatability concerns.

This concept may be useful for early inspiration, but it is not a strong final architecture for a controlled football launcher.

Key risks:

- Poor repeatability.

- Arm impact and rebound hazards.

- Difficult release timing.

- Poor spiral generation.

- Large swept volume requiring guarding.

Mitigations:

- Use only as a low-fidelity comparison.

- Avoid carrying forward unless all higher-scoring concepts become infeasible.

Concept E — Belt/roller acceleration tunnel

Concept E has good theoretical performance because the ball can remain in contact with driven surfaces for a longer time. This could improve speed and spin control. It also offers strong CAD and mechanical design content because it involves rollers, belts, supports, alignment and guarding.

Its weakness is integration complexity. A football has a changing cross-section along its length, so a roller tunnel may need compliant mounts, shaped guides or multiple adjustable contact points. This raises cost, CAD burden and risk of jamming.

Key risks:

- High part count.

- Alignment sensitivity.

- Ball jamming or excessive deformation.

- Higher cost.

- Difficult assembly and guarding.

Mitigations:

- Keep as a future development option.

- Use lessons from it to improve the contactor design in Concept A.

5.6 Selection Decision

The selected concept is:

Concept A — Elastic rail carriage + three-contactor spin module.

This concept scores highest because it best satisfies the football-specific requirements: controlled forward launch, spiral spin, nose-forward release, adjustable contactor preload, repeatable testing and strong CAD/mechanical design evidence. It also offers the strongest portfolio value because it is not just a generic launcher; it separates forward-speed generation from spin generation and creates clear subsystems for analysis, design, manufacture and verification.

The decision is not risk-free. Concept A is more complex than the dual-wheel launcher and will require careful management of stored energy, release timing, carriage arrest, alignment, guarding and contactor preload. However, these risks are visible and can be addressed through the existing V-model, RFLP structure and staged test plan. The concept should therefore move forward only after first-order engineering calculations confirm that the elastic energy, launch force, motor speed, contact preload and carriage arrest loads are plausible.

5.7 Architecture Freeze Conditions

Concept A should not move directly into detailed CAD until the following items are checked in Phase 6:

| Check | Required outcome |
| --- | --- |
| Launch energy budget | Stored elastic energy can plausibly produce 10–20 m/s exit speed without excessive force. |
| Elastic drive sizing | Spring/elastic force and carriage travel are realistic for the frame and latch. |
| Spin module budget | Motor speed, contactor RPM and contact duration can plausibly produce 600–1000 rpm ball spin. |
| Contact preload check | Adjustable preload range is enough to prevent slip without excessive ball deformation. |
| Carriage arrest budget | End-stop or bumper can absorb residual carriage energy safely. |
| Stability check | Frame will not tip, slide or twist during launch. |
| Safety check | Guarding, emergency stop and release inhibit can be implemented at prototype level. |

5.8 Phase 5 Exit Gate

Phase 5 is complete when the concept selection matrix, concept assessment and selected architecture are recorded. The selected concept is Concept A, but the choice remains provisional until Phase 6 confirms the first-order engineering budgets. If the calculations show that Concept A requires unrealistic launch force, unsafe stored energy, excessive contact preload or unmanageable carriage arrest loads, Concept B should be reconsidered as the lower-risk backup concept.